| County: |  |
| --- | --- |
| Federal Project No.: |  |
| State Project No(s): |  |
| Bridge Repair Project No.: |  |
| P.E. No.: |  |
| PIN: |  |
| Description: |  |
| TMP Completion Date: |  |
| Submitted by Designer: (TDOT or Consultant) Date: |  |
| ALL ITEMS HAVE BEEN REVIEWED AND DETERMINED TO BE READY FOR SUBMITTAL BY: |  |
| Name of TDOT Project Manager: Date: |  |
| Comments: |  |

RESURFACING INDEX OF SHEETS

SIGNATURE SHEETS 	ROADWAY-SIGN1

TITLE SHEET	1

ROADWAY INDEX, STANDARD ROADWAY DRAWINGS, AND STANDARD

TRAFFIC DESIGN DRAWINGS 	1A, 1A1, 1A2

PROJECT COMMITMENTS 	1B

ESTIMATED ROADWAY QUANTITIES	2, 2-1, 2-2

TYPICAL SECTIONS AND PAVEMENT SCHEDULE 	2B, 2B1, 2B2

GENERAL NOTES	2C, 2C1

SPECIAL NOTES	2D, 2D1

ENVIRONMENTAL NOTES	2E, 2E1

TABULATED QUANTITIES	2F, 2F1, 2F2

DETAIL SHEETS	2G, 2G1, 2G2

UTILITY NOTES AND UTILITY OWNERS	3

RAILROAD LAYOUT	4, 5, 6

TRAFFIC CONTROL PLANS	T1, T2, T3

BRIDGE REPAIR PLANS	B-1

For items not needed, do not include in index, do not renumber sheets for omitted items.

Figure 1: Index of Sheets

This checklist will be submitted as part of the PS&E Reports folder in the Final PS&E submittal package and must be completed in its entirety.  (see FINAL PREPARATION OF RESURFACING PLANS)

For more information related to Resurfacing plans, including example sets, letting submittal schedules, seed files, etc., see the Resurfacing Project Selection and Development webpage.

The checklist is written to clearly define features and text that shall be shown on Resurfacing Plan sheets to ensure there is consistency throughout the state. Information can be found in the CADDV8 document.  If any of the items are not applicable to your project, then do NOT include and mark N/A. If there are questions, contact your Design Manager.

For further explanation of defined features, see CADDV8.pdf.

To aid Designers in the creation of the Title Sheet and to ensure the correct features are shown, a sheet level filter has been provided in OpenRoads Designer (ORD). The Designer shall not turn on levels for features at the request of others.

Information for each sheet shall be filled in correctly in the upper right corner including TYPE (RESURF.), LETTING YEAR, FEDERAL or STATE PROJECT NUMBER, and SHEET NUMBER. TDOT PROJECT SEARCH shall be checked for possible changes to project numbers prior to Final PS&E Plan submittal.

Sheet scales for all sheets are set by the seed file used to create that sheet.

- Seed2D yields an active scale of 1” = 50’.  This is used for Curb Ramp Details, Railroad Recommendation Details, Railroad Layout, and other similar sheets.

- English General Notes, Index and Standard Drawings, and other similar sheets yield an active scale of 1” = 1’.

- Some 2nd sheets like Typical Section sheets are not drawn to scale but shall still use an approved sheet border.

All plan sheets shall be signed for Final PS&E submittal with the following exceptions:

Curb Ramp sheets provided by Curb Ramp Consultants and/or Pavement Marking sheets provided by TDOT Traffic Operations, Local Agencies, or their representative.  These 2G series sheets shall be marked “FOR INFORMATION ONLY” and will not be signed.

SIGNATURE SHEET

1		ROADWAY-SIGN1

A signature sheet shall be used for engineers to digitally sign the plan set. An index of sheets should be below each signature listing the sheets that the engineer is responsible for the design.  Refer to the Digital Signature Certification Workflow document for additional information.

SHEET 1 SERIES

An ORD title sheet template is provided in a seed file format.  All items listed below are in a data field or in a box with levels that can be turned on or off as needed.

1		TITLE SHEET

| YES | N/A |
| --- | --- |
| ☐ | ☐ | For projects to be let/bundled with another project, include bundle notes on the left side of the page below the Index of Sheets. Include PIN(s), route(s), and county(ies) of bundled projects. Projects let with/bundled with another project shall be a separate set of plans and quantities. |
| ☐ | ☐ | Begin/End Resurfacing or Resurfacing & Safety project limit labels/flags with applicable Resurfacing and Safety Federal and/or State project number(s), termini description and LM. |
| ☐ | ☐ | For projects with Bridge Repair, include a label/flag for bridge location(s), bridge number(s), and state Bridge Repair project number. Include leaders for all applicable structures. See TDOT Project Search for log miles. |
| ☐ | ☐ | For projects with Railroads, include labels/flags for crossing location(s) showing the Railroad Name, Crossing type & Number, Route Log Mile, Railroad Milepost, and Latitude/Longitude. This information can be located in E-Trims and via Federal Railroad Administration Office of Safety Analysis. |
| ☐ | ☐ | For all projects, check E-Trims for traffic counters, weather stations, embedded detection loops & automatic traffic readers – add these items to the applicable block with item identification and log mile location. |
| ☐ | ☐ | Bridge I.D.(s) identified above or below state map |
| ☐ | ☐ | Chapter 86 toggled, Yes or No |
| ☐ | ☐ | Work Zone Significance toggled, Yes or No |
| ☐ | ☐ | Railroad Involvement toggled, Yes or No |
| ☐ | ☐ | County or Counties patterned on the state map |
| ☐ | ☐ | Design Traffic Data Table filled in/updated to letting year and projected volumes. List log mile ranges where speed changes (if applicable) |
| ☐ | ☐ | Engineer’s seal with signature and date for Final PS&E submittal |
| ☐ | ☐ | Plan Phase Stamp (Plan-in-Hand Field Review) for PIH submittal |
| ☐ | ☐ | Identification block in lower left-hand corner completed with PE-D project number and label (Design) and PIN for project, fill in TDOT Project Manager, Designer, and Checked by data fields. For consultant projects, fill in TDOT Project Manager, Designed by Consultant Firm, Designer, and Checked By data fields. |
| ☐ | ☐ | Location map showing route to be improved, local roads, streams, railroads, and towns |
| ☐ | ☐ | Map Scale |
| ☐ | ☐ | No Exclusions block or Exclusions block with Log Mile range(s) identified |
| ☐ | ☐ | North arrow |
| ☐ | ☐ | Project Description filled in under County/Counties. Description shall match TDOT Project Search including interstate, state route, or local road name and project limits. |
| ☐ | ☐ | Type of work shall be Resurface and Safety or Resurface as shown in TDOT Project Search |
| ☐ | ☐ | Type of work should be further identified (i.e., Thin Lift D, Pavement Markings, Cold Planing, Curb Ramp, Guardrail, etc.). |
| ☐ | ☐ | Identify State Route and US route numbers (if applicable). |
| ☐ | ☐ | Resurfacing project length, Lane Miles Paved (truncate to three (3) decimals - no rounding). |
| ☐ | ☐ | “Index of Sheets See Sheet No. 1A” added in upper left corner |
| ☐ | ☐ | Sheet title block in upper right corner filled in with letting year, sheet number “1”, Federal Project Number, and State Construction Project Number(s) |
| ☐ | ☐ | Signatures of Chief Engineer and Commissioner in signature block |

1A		ROADWAY INDEX, STANDARD ROADWAY DRAWINGS, AND STANDARD TRAFFIC DESIGN DRAWINGS

See Pavement Program website for Resurfacing Index & Standard Drawings.

| YES | N/A |
| --- | --- |
| ☐ | ☐ | Resurfacing Index of sheets (See Figure 1: Index of Sheets) |
| ☐ | ☐ | Include a note below the index of sheets that states “NO UTILITY SHEETS ARE INCLUDED IN THIS PLAN SET.” |
| ☐ | ☐ | Standard Roadway Drawings listed (check for additional or revised drawings including descriptions and dates as listed in current instructional bulletins) |
| ☐ | ☐ | Standard Traffic Design Drawings listed (check for additional or revised drawings including descriptions and dates as listed in current Traffic Operation memorandum) |

1B		PROJECT COMMITMENTS

| YES | N/A |
| --- | --- |
| ☐ | ☐ | Check for commitments in TDOT Project Search prior to Plan-in-Hand Field Review submittal. If a commitment is pending at the time that plans are submitted, contact the Division responsible for the commitment to get an update on when or/if it may become active. If a commitment is added after Plan-in-Hand submittal, it will be added to the PS&E Plan set prior to submittal. |

SHEET 2 SERIES

QUANTITIES

All quantities shall be approved TDOT items, descriptions, and units as listed in the Items.dat file. See Chapter 1 of the Roadway Design Guidelines for additional information. Curb Ramp, Pavement Marking and other items may be provided by others but shall be included in the plans.  Structures Division is responsible for creating an estimated quantities sheet for inclusion in the Bridge Plans (B-1).

2		ESTIMATED ROADWAY QUANTITIES

(2-1, 2-2, etc. if needed)

| YES | N/A |
| --- | --- |
| ☐ | ☐ | Check for removal item numbers (examples: guardrail, signs, curb ramps) |
| ☐ | ☐ | Estimated Roadway Quantities Block with Item Number, Description, Unit and Quantity filled in from most recently updated Excel file |
| ☐ | ☐ | Footnotes (add for clarity such as placement, payment, or when used in multiple locations, etc., add to define when substitutions are acceptable, and add to identify maintenance schedules or cycles, etc.) Check against footnotes on tabulation blocks in the 2F Series sheets |
| ☐ | ☐ | Footnotes as specified in Roadway Design Guidelines for certain items |
| ☐ | ☐ | Footnotes shall use numbers and be shown in numerical order. Letters from the alphabet shall not be used. |
| ☐ | ☐ | Item Nos listed in numerical order |
| ☐ | ☐ | Multiple quantity columns for separating resurfacing and safety quantities (if applicable) |

2B		TYPICAL SECTIONS AND PAVEMENT SCHEDULE

(2B1, 2B2, etc. if needed)

The following checks apply to the mainline. Each typical section shall be defined by route name and log mile limits.

| YES | N/A |
| --- | --- |
| ☐ | ☐ | Mainline typical section(s): label widths for applicable elements (travel lanes, turn lanes, shoulders) and LM ranges for each typical |
| ☐ | ☐ | Pavement Schedule block shown on the last page of the typical section sheet series with pavement layers including depth and applicable rates defined. Rates shall be checked against current Roadway Design Guidelines for possible changes. Each layer shall be coded on the typical section(s). (this information should be verified with TDOT Project Search and the Materials & Test recommendations and should include alternates, if applicable). |
| ☐ | ☐ | Special details (taper milling, transition details, spot leveling, full depth repair/breakouts, partial depth asphalt repair, OGFC overlay for concrete ramps, or other project specific details) |

2C		GENERAL NOTES

(2C1 if needed)

See Pavement Program website for Resurfacing General Notes.

| YES | N/A |
| --- | --- |
| ☐ | ☐ | Check all notes for referrals to Standard Roadway Drawings. Ensure all Standard Drawings are shown in the Standard Drawings list and reference the most current drawing |
| ☐ | ☐ | Check all notes for referrals to pay items. Ensure all pay items are included in the Estimated Roadway Quantity sheet and in tabulation blocks |
| ☐ | ☐ | Description blanks in notes shall be populated |
| ☐ | ☐ | Update notes prior to submittal by checking recently issued Instructional Bulletins |

2D		SPECIAL NOTES

(2D1 if needed)

See Pavement Program website for Resurfacing Special Notes.

| YES | N/A |
| --- | --- |
| ☐ | ☐ | Check all notes for referrals to Standard Roadway Drawings. Ensure all Standard Drawings are shown in the Standard Drawings list and reference the most current drawing |
| ☐ | ☐ | Check all notes for referrals to pay items. Ensure all pay items are included in the Estimated Roadway Quantity sheet and in tabulation blocks |
| ☐ | ☐ | Description blanks in notes shall be populated |
| ☐ | ☐ | Special notes shall be added if specific to the job |
| ☐ | ☐ | If there is railroad involvement, include special notes or other information provided by the railroad(s) |
| ☐ | ☐ | Update notes prior to submittal by checking recently issued Instructional Bulletins |

2E		ENVIRONMENTAL NOTES

(2E1 if needed)

See Pavement Program website for Resurfacing Environmental Notes.

| YES | N/A |
| --- | --- |
| ☐ | ☐ | Check all notes for referrals to Standard Roadway Drawings. Ensure all Standard Drawings are shown in the Standard Drawings list and reference the most current drawing |
| ☐ | ☐ | Check all notes for referrals to pay items. Ensure all pay items are included in the Estimated Roadway Quantity sheet and in tabulation blocks |
| ☐ | ☐ | Description blanks in notes shall be populated |
| ☐ | ☐ | Scope of Work shall be shown with description of project overview including type of work to be performed, construction details, any project specific expected goals, requirements, and/or limitations that are not project commitments. |
| ☐ | ☐ | Special notes shall be added if specific to the job |
| ☐ | ☐ | Update notes prior to submittal by checking recently issued Instructional Bulletins |

2F		   TABULATED QUANTITIES

(2F1, 2F2, etc. as needed)

Note to Designer: Designer shall use available formatted tabulation quantity blocks applicable to the project as provided in the Estimate spread sheet template. Remove any item columns not used to avoid confusion.  When standardized tabulation blocks are used, it is easier to understand where quantities were calculated and ensures the Designer does not omit necessary pay items on the Estimated Quantity sheet. Tabulation blocks with “Remarks” columns shall be filled in for clarity.

| YES | N/A |
| --- | --- |
| ☐ | ☐ | Bridge Recommendations – if there are no bridges on the project, add the block and the statement “THERE ARE NO BRIDGES WITHIN THE PROJECT LIMITS” |
| ☐ | ☐ | Guardrail Tabulation (Resurfacing) |
| ☐ | ☐ | Storm Drain Adjustments (Resurfacing) |
| ☐ | ☐ | Signal Quantities |
| ☐ | ☐ | Ramp Acceleration/Deceleration Tabulation |
| ☐ | ☐ | Traffic Control Sign Tabulation (Resurfacing) |
| ☐ | ☐ | Utility Adjustments (must include utility owner) |
| ☐ | ☐ | Curb Ramp Tabulation |
| ☐; ☐; Ramp, Acceleration/Deceleration Tabulation. | ☐; ☐; Ramp, Acceleration/Deceleration Tabulation. | Partial Depth and/or Full Depth Repair Tabulation. |

2G		DETAIL SHEETS

(2G1, 2G2, etc. if needed)

The following sheets shall be included as needed in the order specified below.  Items shall be grouped when multiple sets are required using sheet # of # for each.

| YES | N/A |
| --- | --- |
| ☐ | ☐ | Curb Ramp Details |
| ☐ | ☐ | Pavement Marking Details |
| ☐ | ☐ | Railroad Crossing Details. |
| ☐; ☐; Pavement Marking Details. | ☐; Pavement Marking Details. | Traffic Detection Loop Details |

3		UTILITY NOTES AND UTILITY OWNERS

| YES | N/A |
| --- | --- |
| ☐ | ☐ | Utility Notes |
| ☐ | ☐ | Utility Owner names with contact information (provided by Regional Utility personnel or others) |

4		RAILROAD LAYOUT

(5, 6, etc. if needed)

Note to Designer: If the project involves Railroad property coordination (Fee or Easement), layout sheets from those plans shall be included and may be obtained from the Design Manager overseeing the project.  These sheets shall include survey or aerial layout showing railroad rights-of-way, easements and air rights at each crossing per the property coordination plans.

T1			TRAFFIC CONTROL PLANS

(T2, T3, etc. if needed)

Note to Designer: If project specific traffic control plans are necessary, including traffic phasing notes, legend, and tabulation, please refer to the Roadway Design Guidelines Chapter 6 Plan-in-Hand checklist for additional information.  If no additional traffic control notes or drawings are required, Pavement Edge Drop-Off Notes for Traffic Control may be reduced to only applicable notes and added at the end of the Special Notes.

| YES | N/A |
| --- | --- |
| ☐ | ☐ | Pavement edge drop-off notes – T1 |
| ☐ | ☐ | Traffic control notes, legend, tabulation – T2 |
| ☐ | ☐ | Traffic control plans – T3, T4, etc. |

B-1		BRIDGE REPAIR PLANS

If proposed bridge repairs requiring a plan set are included in the project, sheets shall be completed by the Structures Division and provided to the Design Manager overseeing the project for insertion into the Final PS&E package as described in Final Preparation of Resurfacing Plans. The Structures Division shall seal the sheets for Resurfacing Submittal and follow the Resurfacing submittal process.  The first sheet of the Bridge Repair Plans, B-1, will contain an index for the rest of the sheets in the Bridge Repair series.  The estimated bridge repair quantities shall also be a part of the Bridge Repair series.

FINAL PREPARATION OF RESURFACING PLANS

|  | This section of the checklist is written to clearly define which documents are included in the Final PS&E (Complete Package) Submittal to ensure there is consistency throughout the state. Information can be found in the Roadway Design Guidelines - PDN. If any of the items are not applicable to your project, then do NOT include and mark N/A. If there are questions, contact your Regional Roadway Team Lead or Project Manager. For further explanation of defined features, see the Roadway Design Guidelines - PDN or the Project Delivery Network Manual. TDOT Project Search shall be checked for possible changes to project numbers prior to Final PS&E submittal. |  |  |
| --- | --- | --- | --- |
| Design Files For the Design Files submittal, the folder of design files submittals shall be added to the zipped Final PS&E Submittal Package. This is in the form of a folder with individual files added. This folder will contain the project’s entire WorkSet folder, acquisition table Excel file, quantities Excel file, etc. The naming convention for the submittal package shall be nnnnnn-nn-DesignFiles. Refer to RDG Chapter 1-105.10 or the PDN 4RD1 for more information. |  |  |

| Yes | N/A |  |
| --- | --- | --- |
| ☐ | ☐ | ORD WorkSet Folder/project related DGN files |
| ☐ | ☐ | Roadway Estimate Excel File (.xlsm) |
| ☐ | ☐ | Other (Description) |

| Estimates For Final PS&E Estimates submittal, the folder of estimate submittals shall be added to the Zipped Final PS&E Submittal Package. This is in the form of a folder with individual files added. The order structure and naming convention is shown in Figure 1-11A, Estimates PS&E Submittal Example. The naming convention for the submittal package shall be nnnnnn-nn-Estimates. Refer to RDG Chapter 1-105.10 or the PDN 4RD1 for more information. |  |  |
| --- | --- | --- |

| Yes | N/A |  |
| --- | --- | --- |
| ☐ | ☐ | Roadway PS&E Estimate |
| ☐ | ☐ | Structures PS&E Estimate |

Final PS&E Plans The Final PS&E plans shall include all items listed in the Roadway Design Checklist for PS&E document turn-in, including, if applicable, bridge plans. These plans shall be uploaded to FileNet/SharePoint/ProjectWise by the respective Disciplines for use by the Design Lead to include in the submittal package. All items listed below should be addressed before compiling the plan sets.

| YES | N/A |  |
| --- | --- | --- |
| ☐ | ☐ | It is the responsibility of the Designer to review the final plans to ensure that sheet numbers are correct according to the index and are in order; pdf pages rotated correctly; PDF sheet sizes for pdfs created from ORD are correct according to current CADD sheet sizes; PDF sheet sizes for scanned in sheets are 34” x 22”; and indexes set up correctly prior to turn in. |
| ☐ | ☐ | Check TDOT Project Search for any changes to the R.O.W. and Construction project numbers on all PS&E plan sheets and project commitments |
| ☐ | ☐ | Check the latest Instructional Bulletins to ensure that there is no change in standard drawings and/or Design Guidelines that may affect the plans. |
| ☐ | ☐ | Final PS&E plans shall address all comments received at the PIH Field Review, and those received from Regional Quality Team. |

For Final PS&E Plans submittal, the plan files shall be added to the zipped Final PS&E Submittal Package.  This is in the form of an Adobe Portfolio PDF with individual files added.  The order structure and naming convention is shown in Table 1-12, Final PS&E Plans Submittal Package List.  See Figure 1-12, Final PS&E Plans Submittal Package Example.  The naming convention for the submittal package shall be nnnnnn-nn-FinalPS&E.pdf. Refer to RDG Chapter 1-105.09 or the PDN 4RD1 for more information.

| Yes | N/A |  |
| --- | --- | --- |
| ☐ | ☐ | Complete sealed set of Resurfacing Plans |
| ☐ | ☐ | Bridge Plans, as applicable |
| ☐ | ☐ | Bridge Reference Documents, as applicable |

Flattened Plan Set

For the flattened Final PS&E Plans document, create a flattened PDF with all disciplinary plans combined into one PDF. Follow this guide when Creating the Flattened Plan Set. This flattened plan set shall include plan sets from the following disciplines, if applicable: Roadway Design, Structures (includes reference documents). See Table 1-12A, Flattened Plan Set List, and Figure 1-13, Flattened Plan Set Example for order structure and naming convention. The naming convention for the submittal document shall be nnnnnn-nn-FinalPS&E-Flattened.pdf.  Refer to RDG Chapter 1-105.09 or the PDN 4RD1 for more information.

| Yes | N/A |  |
| --- | --- | --- |
| ☐ | ☐ | Complete sealed set of Resurfacing Plans |
| ☐ | ☐ | Bridge Plans, as applicable |
| ☐ | ☐ | Bridge Reference Documents, as applicable |

Reports

For Final PS&E Reports submittal, the folder of all reports that are required for the project shall be added to the zipped Final PS&E Submittal Package.  This is in the form of a folder with individual files added.  The reports that may be included shall be the special provisions order, PDFR, Bridge Recommendations, etc. See Table 1-12B, Reports Submittal Package List Example. The naming convention for the submittal package shall be nnnnnn-nn-Reports. Refer to RDG Chapter 1-105.09 or the PDN 4RD1 for more information.

| Yes; N/A. ☐; ☐; Environmental Permits – or notice that No permit required (pdf). ☐; ☐; RR Agreement – SP 105C (pdf). ☐; ☐; ROW Availability – SP 107C (pdf). ☐; ☐; Asbestos Report (pdf). ☐; ☐; Special Provision Order Sheet (.xlsx). ☐; ☐; Bridge Inspection Reports as Applicable (pdf). ☐; ☐; Signed Traffic Management Plan (pdf). ☐; ☐; Project Notebook (xlsm). ☐; ☐; Signed Resurfacing Checklist (pdf). |  |  |
| --- | --- | --- |

FINAL PS&E PACKAGE

For Final PS&E Package submittal, the complete package submittal shall be added to FileNet or ProjectWise. This is in the form of a zipped folder with individual files added.  The order structure and naming convention is shown in Figure 1-14, Zipped “Complete Package” Final PS&E Submittal Package Example. The naming convention for the submittal package shall be nnnnnn-nn-FinalPS&EPackage.zip. Refer to RDG Chapter 1-105.09 or the PDN 4RD1 for more information.

| Yes | N/A |  |
| --- | --- | --- |
| ☐ | ☐ | Construction Letting Files Folder |
| ☐ | ☐ | Design Files Folder |
| ☐ | ☐ | Estimates Files Folder |
| ☐ | ☐ | Reports Files Folder |
| ☐ | ☐ | Final PS&E – Flattened (pdf) |
| ☐ | ☐ | Final PS&E (pdf) |