Vehicle fleet

Maintenance Plan

___________________________                                             ____________________

Authorized signature					Date of Approval

(a representative of the governing board or director(s))

# (Agency Name)

(List the Complete Address, phone number and email for Agency)

# Vehicle fleet maintenance plan

# Introduction

(Agency Name) in its continuous development and concern for the safety of its staff and community members has developed this maintenance plan. This is a living document that will be updated on an “as needed” basis and reviewed annually for compliance to new rules, regulations, and laws.

This plan is designed to keep all vehicles and related equipment in safe, reliable, and operational condition. It requires management, drivers, and related staff to be well trained and accountable for specific roles.

SPECIFIC ROLES

### Management

(Agency, Position Title of Applicable Staff) will make sure that all staff is properly trained and certified as deemed appropriate to perform preventive maintenance on the vehicles and will document all maintenance related activities.

### Drivers

The drivers must be certified according to State laws. Driver must know the proper starting, shifting, and braking procedures to extend the life of the vehicle and must be vigilant in reporting their observations. No vehicle should be sent into service low on oil, antifreeze, automatic transmission, or power steering fluid. Unsealed batteries and windshield washer fluid must also be checked and filled. Drivers should be alert for unusual noises, bad tires, noisy or poor brakes, and clutch adjustments.

All drivers should be completely familiarized with the vehicles including engine compartment, driver controls, and passenger safety devices. Drivers should be trained to recognize unusual noises and describe basic mechanical problems to the supervisor and/or mechanic.

# Vehicle FLEET Maintenance

# Goals and Objectives

(Agency Name) will use the maintenance plan to obtain the goals listed below:

1)

2)

Our objectives to complete the goals listed above include:

1)

2)

3)

4)

(Agency Name) has the means to carry out the goals and objectives by:

1)

2)

# General Agency and vEHICLE fLEET iNFORMATION

(Describe the type of vehicles in the fleet, the basic kind of transportation provided by the Agency, basic area that is covered, specific location(s) where the vehicle(s) is parked, and state how many vehicles are ADA accessible.)

Sample language, tailor to your agency: (Agency) has 12 group homes in (X) County, and vehicles are located at each home to provide transportation to residents for medical appointments, shopping, and recreational outings. The vehicles are transit vans that are ADA accessible except for one non-accessible minivan that serves a home with only ambulatory residents. OR (Agency) has a day center located in McMinnville, TN, and has a fleet of three ADA accessible transit vans at the center that provide transportation for clients to and from the center and for recreational outings throughout Warren County.

# Preventive maintenance inspections & services

## Introduction

Vehicle and component (e.g., wheelchair lift) manufacturer manuals are an important part of the vehicle maintenance plan as they define specific maintenance intervals and provide critical information when the maintenance work is actually to be performed.

Preventive maintenance (PM) inspections and scheduled services should follow the intervals recommended by the manufacturer. If preventive maintenance services are not being done according to the guidelines of the manufacturer, the agency may jeopardize any claim to a warranty.

(See Appendix E: Preventative Maintenance Schedule Chart)

WARRANTY RECOVERY

Services eligible for warranty payment must be made by the appropriate personnel and filed with the manufacturer. Documentation of such services should remain in the vehicle file.

(Agency Name) operates a warranty recovery program to ensure that cost of parts and repairs on warranty-covered items are recovered.

Authorization for warranty return and labor claims, if applicable, are obtained from the manufacturer or vendor. Information is supplied to the vendor on the circumstances of the failure, if known. The item is then returned to the vendor warranty department for repair or replacement. (Agency Name) retains copy of the warranty claim form for tracking purposes. The Agency will also notify TDOT of all warranty returns.

## Documentation

Preventive maintenance (PM) inspections and scheduled services should be performed, and documented according to a proper schedule. All documentation including maintenance forms, logs, receipts, inspections, and trip logs should be kept through the life of the vehicle plus 3 years. Whenever a mechanic or tow truck is dispatched to a vehicle in service, documentation should be submitted and placed in the vehicle file. (Agency, Position Title of Applicable Staff) is responsible for maintaining the vehicle documentation.

(See Appendix A: Information for Onsite Mechanic/Tow Sheet)

## Preventative Maintenance Inspections

Preventive maintenance (PM) inspections are basic inspections to help provide an opportunity to detect and repair damage or wear conditions before major components need repairs. Such inspections occur pre- and/or post-trip, and at scheduled intervals.

These inspections generally cover the following elements:

- A list of specific items to be checked

- Record repairs and the routine application of fluids

- Indicate inspection interval and mileage

- Indicate if repair/replacement needed for an item.

## Pre/Post Trip inspections

An important aspect of preventive maintenance is the establishment of strong communication between drivers and management. An easy way to ensure and document this communication link is through the use of the driver’s daily vehicle inspection checklist that is either a pre-trip or post-trip inspection.

The driver should identify any defects and report them to (Agency, Position Title of Applicable Staff). (See Identified Defects section below.) All checklists are to be maintained in the vehicle’s permanent file.

The pre-trip or post-trip inspection forms shall be legibly completed and signed by the vehicle driver.

(See Appendix B: Pre-/Post-Trip Inspection Checklist)

A pre-trip or post-trip inspection should include at a minimum:

- Cleanliness – Properly maintained and free of loose articles.

- Lights and reflectors – High/low beams, tail lights, turn signals, 4-way hazard flashers, marker lights, license plate light, and reflectors should be cleaned as needed

- Brakes – Both foot and emergency brakes should be capable of effectively stopping or restraining the vehicle. Brake pedal should be firm after 1-2 inch free-play on a single down stroke. No noises, vibration or steering changes should result from applying the brakes while moving.

- Horn – Gives an adequate and reliable warning signal.

- Windshield, washer, wipers and defroster – Surfaces must be clean and unobstructed, inside and outside. Washer reservoirs are to be filled as needed.

- Mirrors – All rear vision mirrors must be clean, properly adjusted and unobstructed. Outside mirrors must be mounted on both sides.

- Tires – Must be of adequate load capacity when vehicle is fully loaded. Tires shall be inflated to recommended pressures and compatible with each set (i.e., all radials or all bias ply; no mixed sets). Tire wear surfaces and sidewalls shall be inspected daily for debris, damage, and wear. Tires shall be replaced prior to revealing the “wear bars” between the treads at the contact surface.

- Speedometer – Shall be operational and accurately record speed.

- Seat Belts – If the vehicle has seat belts, they must be in good operating condition and used by all passengers and drivers. Wheelchair passenger restraints and securement systems shall be fully operational.

- Doors – Capable of being opened, shut, and locked as required.

- Fluids – All fluid levels must be checked each time the vehicle is fueled and maintained at the manufacturers recommended operating levels. This includes engine coolant, oil, brake fluid, power steering fluid, transmission fluid, and washer solvent.

- Wheelchair lifts – Check operating and structural condition by operating through one complete cycle.

- Emergency Equipment – Should be present and operational:

| - Flares | - Fire Extinguishers |
| --- | --- |
| - First Aid Kits | - Flashlight W/Batteries |
| - Bloodborne Pathogens Clean-Up Kit | - Reflective Triangle |
| - Reflective Vest for Driver | - Clean-Up Kit for Cleaning & Sanitizing the Vehicle |
| - Seatbelt Cutter |  |

The inspection shall identify the vehicle and list any defect or deficiency discovered by or reported to the driver which would affect the safety of operation of the vehicle or result in its mechanical breakdown. If no defect or deficiency is discovered by or reported to the driver, the report shall so indicate. In all instances, the driver shall sign the vehicle inspection report.

## Identified defects

Identified defects should be reported to (Agency, Position Title of Applicable Staff). Defects must be reviewed and repaired based on the categories listed:

### Safety defect

The vehicle cannot be released until the repairs are completed, except in case of an emergency. Safety cannot be compromised.

### Mechanical defect

A defect that will worsen and increase cost. The vehicle cannot be released until the repairs are completed, except in case of an emergency.

### Elective mechanical defect

A defect that does not compromise safety and will not cause further damage if operated but needs to be corrected prior to the next PM cycle. Repair should be scheduled. Due to transportation costs and disruption to operations, this decision should not be made lightly.

### elective or cosmetic defect

The defect will not compromise safety and will not cause further damage or cost as it is an aesthetic defect. The vehicle should be scheduled for an off-peak time in the future, as determined by management, or at the next scheduled PM Service.

(See Appendix C: Defect Report)

## interval inspections and service

The manufacturer’s recommended service schedule should be adhered to by either mileage or months. Basic PM Services are 4 levels that are listed below:

(Note: See Manufacturer’s Manual for mileage/month intervals)

Level A – Conducted at (? Miles/ ? months interval). Change oil and filter, inspect tires, electrical system, service all fluid levels, lubricate chassis and doors, check A/C, hoses, fire extinguishers, belts, brakes, lights, test drive, body damage, etc.  Inspect and test vehicle lift.

Level B – Conducted at (? Miles/ ? months interval). Includes all items in level A. Check coolant, specific gravity, and ph.

Level C – Conducted at (? Miles/ ? months interval). All items in levels A and B, plus change fuel filter, replace air filter, and inspection of braking system.

Level D – Conducted at (? Miles/ ? months interval). All items in levels A, B, and C, plus inspection and repack of wheel bearings.

(See Appendix D: PM Service Schedule sheet and Appendix E: PM Schedule Chart)

# ADA Accessibility Equipment

## introduction

Title 49 CFR Part 37 Transportation Services for Individuals with Disabilities (ADA) Subpart G Section 37.161 requires that transportation services maintain the ADA features of their facilities and vehicles in operative condition. These ADA features, include, but are not limited to:

- Lifts and other means of access to vehicles;

- Securement devices;

- Signage or systems to aid communications with persons who have impaired vision or hearing.

Accessibility features must be repaired promptly if they are damaged or out-of-order. When an accessibility feature is out-of-order, (Agency Name) shall take reasonable steps to accommodate persons with disabilities who would otherwise use the feature.

Title 49 CFR Section 37.163 requires the establishment of regular and frequent maintenance checks of the lifts. The vehicle drivers must report, by the most immediate means available, any failure of a lift. The vehicle lift should be repaired as soon as possible.

Title 49 CFR Section 37.173 requires all personnel to be trained to proficiency in the use of ADA equipment, as appropriate to their duties.

## Preventive maintenance plan

A preventive maintenance plan for ADA accessibility features should be in place; including a system of maintenance checks based on manufacturer-recommended guidelines within number of cycles or yearly, whichever comes first.

(See Appendix H: Wheelchair Lift Maintenance Policy)

# MANAGEMENT OF VEHICLE FLEET

### Physical inventory

(Agency Name) will conduct a physical inventory check on all vehicles and lifts annually.

INVENTORY RECORD

(Agency Name) will maintain an ongoing inventory of all grant-funded vehicles, and locally-funded vehicles named to be replaced by grant-funded vehicles, that includes the following elements:

- Description (make, model, year)

- Vehicle Identification Number (VIN)

- Physical Title Holder (should be TDOT for grant-funded vehicles)

- Name on Title

- Acquisition Date

- Active Service Date

- Cost of the Vehicle

- Percentage of Federal participation in the vehicle cost

- Grant source of funding for the vehicle (grant number)

- Current location of the vehicle (at least county) – update as changes occur

- Current use of the vehicle (Active, Out of Service, Disposal in Process, Inactive (disposed)) – update as status changes

- Current condition of the vehicle (Excellent, Good, Fair, Poor) – update annually

- Odometer reading – update annually

- Age of the vehicle – update annually

- Useful life of the vehicle

  - Years

  - Miles

- Disposition data (if applicable):

  - Date removed from service

  - Date of disposal

  - Sale price of the vehicle

### Vehicle history file

Each vehicle will have a written record documenting preventive maintenance, regular maintenance, inspections, lubrications, and repairs performed.

(See Appendix F: Maintenance Log and Appendix G: Mechanic Service Sheet.)

A minimum of the following information will be maintained in the records:

- Identification of the vehicle

  - Serial/VIN Number

  - Year

  - Make

  - Model

  - License Plate Number

- Date of service or inspection

- Mileage at time of service or inspection

- Description of each inspection, maintenance, repair, lubrication performed

- The name of the business/shop performing an inspection, maintenance, lubrication, or repair to the vehicle or lift

(Agency Name) will use the businesses or shops listed below for vehicle maintenance. (Agency Name) has confirmed with the businesses and shops listed below that the mechanics are certified and qualified to perform maintenance work on a vehicle.

1) Name and Full Address Required

2)

3)

(Agency Name) will use the businesses or shops listed below for lift maintenance. (Agency Name) has confirmed with the businesses and shops listed below that the mechanics are certified and qualified to perform maintenance work on a lift.

1) Name and Full Address Required

2)

VEHICLE DISPOSAL

When a grant-funded vehicle is no longer able to be repaired, has been totaled, or is no longer needed for transportation use, (Agency) will contact TDOT to discuss required disposal procedures.

Appendix A:

# Information For Onsite Mechanic/Tow

1) Today’s Date _________________ 	2) Last 5 digits of VIN _________________

3) Time Called : ___________  _____	4) Driver __________________________

5) Route _______________________

6) Location of Vehicle: Be specific – street address, cross street, highway marker

___________________________________________________________________________

___________________________________________________________________________

___________________________________________________________________________

7) Reported Trouble: Ask specific questions and be as precise as possible.

___________________________________________________________________________

___________________________________________________________________________

___________________________________________________________________________

10) Replacement vehicle ____________________	11) Call received by: ____________

### Management REPORT

1) Time arrived at Bus: _________   2) In-Service Repair       Bus Exchange     Towed

(Circle one)

3) Time Repair/Exchange Completed ____________________________________

4) Nature of Trouble _____________________________________________________

______________________________________________________________________

5) Remarks ____________________________________________________________

Operator’s Signature ________________________________

Agency Staff Signature ______________________________

# Appendix B: PRE-/Post-TRIP Inspection Checklist

VIN __________________________		Odometer _______________

Vehicle Number: ___________			Date  ___________________

| 1 | All Seats and Seatbelts |  |  | 20 | Exterior Body And Components |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | Doors/ Hinges/Latches/locks |  |  | 21 | Tires / Wheels – Lug Nuts, Tire Pressure |  |
| 3 | Flooring/Headliner/Side Panels |  |  | 22 | Access Doors/Emergency Doors |  |
| 4 | Mirrors |  |  | 23 | Fuel Cap And Port |  |
| 5 | Interior Lights |  |  | 24 | Engine Oil /Trans. Fluid Check |  |
| 6 | Exterior Lights/Reflectors - Directional - Step/door - Emergency flashers - Clearance - Head lights - Panel lights - Tail lights - Back up lights - Brake lights |  |  | 25 | Power Steering Fluid Level |  |
|  |  |  |  | 26 | Battery |  |
|  |  |  |  | 27 | Radiator Fluid /Coolant Levels |  |
|  |  |  |  | 28 | Belts/Hoses/Wiring |  |
|  |  |  |  | 29 | Under hood/Exhaust System |  |
|  |  |  |  | 30 | Brakes/Brake Fluid/Brake Pedal |  |
|  |  |  |  | 31 | Parking Brake/Emergency Brake |  |
|  |  |  |  | 32 | Acceleration/Steering/Tracking |  |
|  |  |  |  | 33 | Suspension - Shocks/Springs |  |
|  |  |  |  | 34 | Water/Fluid Leaks |  |
| 7 | Warning System/Horn/Radio |  |  | 35 | Ramp - extended |  |
| 8 | Starter System/Automatic Choke/Backup Alarm |  |  | 36 | Wheelchair Lift – Cycled Y/N - Smooth Operation |  |
| 9 | Windshield Wiper/ Washers/ Windshield |  |  | 37 | Interlock System Lift Fluid Levels |  |
| 10 | Windows/Emergency Windows |  |  | 38 | 4 Tie Downs Per Position |  |
| 11 | AC/Heater/defroster – front /rear |  |  | 39 | 4 Min. Safety Loop Strap Per Vehicle /Hand Rails |  |
| 12 | GAUGES: Fuel/Oil/Volt/Temp/Speedometer |  |  | 40 | Flashlight/batteries |  |
| 13 | Roof Hatch |  |  | 41 | Fire Extinguisher /Safety Triangles /Flares /Reflective Vest /Seatbelt Cutter |  |
| 14 | Fare Box |  |  | 42 | Bloodborne Kit /First Aid Kit /Cleaning Kit |  |
| 15 | Clean Interior and Exterior |  |  | 43 | License Plate/Operators Manual |  |
| 16 | Required Stickers/posters displayed |  |  | 44 | Registration/Insurance |  |

| SYMBOLS |  |
| --- | --- |
|  | OK |
| X | REPAIRS REQUIRED |
| R | REPAIRED |
| O | NOT APPLICABLE |

COMMENTS (attach Defect Report if nec.):

Driver: ______________________________

# Appendix C: Defect Report

VIN_____________________			Date ______________________

Mileage ___________________			Please circle all that apply.

| Doors | W/C Lift | A/C or Heat | Exterior Lighting |
| --- | --- | --- | --- |
| Stick | No Power | Defroster | Headlights |
| Too fast | Deploy | No Heat | Tail Lights |
| Too Slow | Platform | No A/C | Turn Signals |
| Won’t Close | Restraint | A/C Light | Flashers |
| Won’t Open | Stow | Blowers | Clearance |

| Electrical | Suspension | Brakes | Body Damage |
| --- | --- | --- | --- |
| Dome Lights | Air Leak | Pull L/R | Bumpers |
| Gauges | Leans | Lock Up | Front End |
| Telltale Lamps | Won’t Raise | Soft | Rear End |
| Horn | Kneeler | Noisy | Left Side |
| Chime | Noisy | Parking Brake | Right Side |

| Windows | Mirrors | Steering | Radio |
| --- | --- | --- | --- |
| Broken | Broken | Hard | Dead |
| Etched | Too Loose | Shimmies | Static |
| Won’t Open | Too Tight | Excessive Play | Volume |
| Won’t Close | Won’t Adjust | Pulls Left | Won’t Transmit |
| Need Cleaning | Spot Mirror | Pulls Right | Won’t Receive |

| Engine |  | Transmission | Tires |
| --- | --- | --- | --- |
| Stop Light | Low Power | Transmission Light | Flat |
| Check Light | Won’t Start | Won’t Shift | Damaged |
| Overheats | Oil Leak | No Forward | Low Air |
| Smokes | Fuel Leak | No Reverse | Low Tread |
| Vibrates | Water Leak | Slips | Uneven Wear |
| Stalls | Noisy | Leaks | Loose Lugs |

| Other Items |  |  |  |
| --- | --- | --- | --- |
| Wipers | Accelerator | Sensitive Edge | Emergency Exits |
| Interior Dirty | Exterior Dirty | Graffiti | Interlock |
| Seats | Other (specify) |  |  |

Identify on page 2 any defect or deficiency discovered that would affect the safety of operation of the vehicle or result in its mechanical breakdown (indicate if none discovered as well): ______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

(Use the back side if more room is needed)

Body Damage (Circle Damage Area(s)

Driver’s Printed Name: ________________________________

Driver’s Signature: ___________________________________

Describe corrective action taken: __________________________________________

_____________________________________________________________________

_____________________________________________________________________

Corrective action. (1) Agency shall certify on the original driver vehicle inspection report which lists any defect or deficiency that the defect or deficiency has been repaired or that repair is unnecessary before the vehicle is operated again.

Print Name of Authorized Agency Staff: ______________________________________

Signature: _______________________________ Date: ________________________

(2) Every transit agency shall maintain the original driver vehicle inspection report, the certification of repairs, and the certification of the driver's review for three months from the date the written report was prepared.

# Appendix D: PM Service Schedule

Preventive Maintenance Level – Schedule by Mileage

VIN _________________________				Date _________________

| PM Level | Cum. Mileage | PM Description | Date of Service | Comments--Note if Added Comments on Back |
| --- | --- | --- | --- | --- |
| A |  |  |  |  |
| A |  |  |  |  |
| A |  |  |  |  |
| B |  |  |  |  |
| A |  |  |  |  |
| A |  |  |  |  |
| A |  |  |  |  |
| C |  |  |  |  |
| A |  |  |  |  |
| A |  |  |  |  |
| A |  |  |  |  |
| B |  |  |  |  |
| A |  |  |  |  |
| A |  |  |  |  |
| A |  |  |  |  |
| D |  |  |  |  |

Repeat the schedule.

Level A – Conducted at ? miles/months interval. Change oil and filter, inspect tires, electrical system, service all fluid levels, lubricate chassis and doors, check A/C, hoses, fire extinguishers, belts, brakes, lights, test drive, body damage, etc.  Inspect and test vehicle lift.

Level B – Conducted at ? miles/months intervals. Includes all items in level A, plus transmission fluid and filter change. Check coolant, specific gravity, and ph.

Level C – Conducted at ? miles/months intervals. All items in levels A and B, plus change fuel filter, perform complete engine tune-up, replace air filter, drain and refill differential lubricant and inspection of braking system.

Level D – Conducted at ? miles/months intervals. All items in levels A, B, and C, plus inspection and repack of wheel bearings.

Appendix E: Preventative Maintenance Schedule chart

| Miles on Vehicle | ? | ? | ? | ? | ? | ? | ? | ? | ? | ? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Months of Vehicle in Active Service | ? | ? | ? | ? | ? | ? | ? | ? | ? | ? |
| Change engine oil & filter | X | X | X | X | X | X | X | X | X | X |
| Rotate tires, inspect tire wear and measure tread depth | X | X | X | X | X | X | X | X | X | X |
| Inspect wheels and related components for abnormal noise, wear, looseness, or drag | X | X | X | X | X | X | X | X | X | X |
| Inspect Wheel-Chair Lift if applicable | X | X | X | X | X | X | X | X | X | X |
| Perform Multi-Point Inspection (Recommended) | X | X | X | X | X | X | X | X | X | X |
| Inspect automatic transmission fluid levels (if equipped with dipstick); consult dealer for requirements |  | X |  | X |  | X |  | X |  | X |
| Inspect brake pads, shoes, rotors, drums, brake linings, hoses, and parking brake |  | X |  | X |  | X |  | X |  | X |
| Inspect engine* cooling system concentration and hoses |  | X |  | X |  | X |  | X |  | X |
| Inspect exhaust system and heat shields |  | X |  | X |  | X |  | X |  | X |
| Inspect front axle and U-joints; lubricate if equipped with grease fittings (4WD vehicles) |  | X |  | X |  | X |  | X |  | X |
| Inspect half-shaft boots (if equipped) |  | X |  | X |  | X |  | X |  | X |
| Inspect steering linkage, ball joints, suspension, tie-rod ends, driveshaft and U-joints; lubricate if equipped with grease fittings |  | X |  | X |  | X |  | X |  | X |
| Torque rear U-bolts (Transit Connect) |  | X |  | X |  | X |  | X |  | X |
| Inspect cabin air filters (if equipped) | X |  | X |  | X |  | X |  | X |  |

(Examples given, but edit/add items according to manufacturer recommendations for actual agency vehicles.)

| Miles on Vehicle | ? | ? | ? | ? | ? | ? | ? | ? | ? | ? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Months of Vehicle in Active Service | ? | ? | ? | ? | ? | ? | ? | ? | ? | ? |
| Change engine oil & filter | X | X | X | X | X | X | X | X | X | X |
| Rotate tires**, inspect tire wear and measure tread depth | X | X | X | X | X | X | X | X | X | X |
| Inspect wheels and related components for abnormal noise, wear, looseness, or drag | X | X | X | X | X | X | X | X | X | X |
| Inspect Wheel-Chair Lift if applicable | X | X | X | X | X | X | X | X | X | X |
| Perform Multi-Point Inspection (Recommended) | X | X | X | X | X | X | X | X | X | X |
| Inspect automatic transmission fluid levels (if equipped with dipstick); consult dealer for requirements |  | X |  | X |  | X |  | X |  | X |
| Inspect brake pads, shoes, rotors, drums, brake linings, hoses, and parking brake |  | X |  | X |  | X |  | X |  | X |
| Inspect engine* cooling system concentration and hoses |  | X |  | X |  | X |  | X |  | X |
| Inspect exhaust system and heat shields |  | X |  | X |  | X |  | X |  | X |
| Inspect front axle and U-joints; lubricate if equipped with grease fittings (4WD vehicles) |  | X |  | X |  | X |  | X |  | X |
| Inspect half-shaft boots (if equipped) |  | X |  | X |  | X |  | X |  | X |
| Inspect steering linkage, ball joints, suspension, tie-rod ends, driveshaft and U-joints; lubricate if equipped with grease fittings |  | X |  | X |  | X |  | X |  | X |
| Torque rear U-bolts (Transit Connect) |  | X |  | X |  | X |  | X |  | X |
| Inspect cabin air filters (if equipped) | X |  | X |  | X |  | X |  | X |  |

(Examples given, but edit/add items according to manufacturer recommendations for actual agency vehicles.)

Appendix F: Maintenance Log for Year: ________

VIN: _____________________________    Plate Number: ____________    Ending Mileage: _________

| Date | Mileage | Garage/Shop Name | Type of Maintenance/Repair Conducted |
| --- | --- | --- | --- |

Additional Comments: _________________________________________________________________

___________________________________________________________________________________

Agency Staff Signature: ___________________________________     Date: _____________________

Appendix G: Mechanic Service Sheet

Date:____________   VIN: __________________________   Plate Number: _______________   Mileage: ________________

Basic Service List

Additional Service List

- Change Engine Oil & Filter

- Tires: Rotate and Measure Tread Depth

- Tire Pressure: RF_____  LF_____  RR_____  LR_____

- Inspect Wheels and Related Components

- Perform Multi-Point Inspection

- Inspect Automatic Transmission Fluid Levels

- Inspect Brake pads, shoes, rotors, drums, brake linings, hoses and parking brake

- Inspect Engine Cooling System Concentration and hoses

- Inspect exhaust system and heat shields

- Inspect front axle and U-joints; lubricate if equipped with grease fittings

- Inspect half-shaft boots (If applicable)

- Inspect steering linkage, ball joints, suspension, tie-rod ends, driveshaft and U-joints; lubricate if equipped with grease fittings

- Torque rear U-bolts

- Inspect cabin air filters

- Lift [Inspect & Lube] if installed in vehicle

- Replace Cabin Air filter

- Replace Climate controlled seat filter

- Replace engine air filter

- Inspect valve clearances

- Change automatic transmission fluid & filter on 5 speed TorqShift

- Replace Front wheel bearing grease

- Replace Spark plugs

- Change engine coolant

- Change manual transmission fluid

- Change rear axle fluid

- Inspect accessory drive belt

- Change front axle fluid

- Change transfer case fluid

- Replace accessory drive belts

- Replace front wheel bearings

Comments:

Parts/Materials Used:

Mechanic Signature: _______________________________________________      Date: _____________________

Appendix H: Wheelchair Lift Maintenance Policy

Wheelchair lifts shall be serviced based on duty cycles. A duty cycle is one full cycle from stowed position back to stowed position. The number of cycles a lift is operated in a given shift will be documented on the daily trip sheet. The supervisor will keep a running tally of the lift cycles for each vehicle, and send the vehicle in for the next designated service prior to reaching the maximum number of lift cycles allowed between services.

There are two different maintenance schedules for wheelchair lifts/ramps: Schedule A (Braun) and Schedule B (Ricon)

Braun: Schedule A: Includes inspection and service at 150, 750, 1,500 and 4,500 cycles with service and inspection at consecutive 750 cycles.

Ricon: Schedule B: Includes inspection and service every 150 cycles, with additional services required at 1,800 cycles. This schedule also requires service at 3,600 cycles to be performed by a certified technician.

Lift Maintenance Schedule A - Braun

Braun 150 Cycles

| Overall condition | Listen for abnormal noises as lift operates: (e.g., grinding or binding noises) |
| --- | --- |
| Control Pendant | Verify that control pendant is undamaged and cable connector is tight. |
| Threshold warning system | Verify that system properly detects objects in threshold area and actuates the audible alarm. |
| Bridge plate load sensor | Verify that sensor inhibits downward movement of platform when a weight is present of lowered bridge plate. |
| Hydraulic fluids | Check for obvious hydraulic leaks |

Braun 750 Cycles

| Inboard roll stop hinge | Apply light oil - See Lubrication Diagram |
| --- | --- |
| Platform hinges (2) | Apply light oil - See Lubrication Diagram |
| Outboard roll stop clevis pin pivot points (4) | Apply light oil - See Lubrication Diagram |
| Outboard roll stop pin roller bearings (2) | Apply light oil - See Lubrication Diagram |
| outboard roll stop foot bearings (2) | Apply light oil - See Lubrication Diagram |
| Outboard roll stop arm slots (2) | Apply light grease - See Lubrication Diagram |
| Lift-Tite™ latches tower pivot points (2 latches-2 points) | Apply light oil - See Lubrication Diagram |
| Lift-Tite™ latch gas (dampening) spring pivot points (2 springs-4 points) | Apply light oil - See Lubrication Diagram |
| Inspect Lift-Tite™ latches and gas speings for wear or damage (bent, deformed or misaligned), positive securement (external snap rings) and proper operation. | Re-secure, replace damaged parts or otherwise correct as needed. Note: Apply light Grease to Lift-Tite™ latch tower pivot point if replacing latch. |
| Inspect inboard and outboard roll stops for proper operation | Correct or replace damaged parts. |
| Inspect outboard roll stop foot pivot for proper operation, positive securement and detached or missing spring. | Correct or replace damaged parts and/or lubricate. See Lubrication Diagram |
| Platform turnbuckle pivot points (2 turnbuckles-4 points) | Apply light oil - See Lubrication Diagram |
| Inspect lift for wear, damage or any abnormal condition | Correct as needed |
| Inspect for rattles | Correct as needed |
| Adjust fold pressure and outer barrier fold pressure (if applicable) | See applicable service manual |

Braun Consecutive 750 Cycle Intervals

Repeat all previously listed inspections, lubrication and maintenance procedures at 750 cycle intervals.

Braun 1,500 Cycles

Perform all procedures listed in previous section

| Upper/lower fold arms (2) | Apply grease (synthetic) to contact areas between upper/lower fold arms. See lubrication diagram |
| --- | --- |
| Platform pivot pin bearings (4) | Apply light oil - See Lubrication Diagram |
| Platform fold axles (2) | Apply light oil - See Lubrication Diagram |
| Inboard roll stop lever bearings (2) | Apply light oil - See Lubrication Diagram |
| Inboard roll stop lever upper slots (2) | Apply light oil - See Lubrication Diagram |
| Saddle support bearings (8) | Apply light oil - See Lubrication Diagram |
| Parallel arm pivot bearings (8) | Apply light oil - See Lubrication Diagram |
| Handrail pivot pin bearings (4) | Apply light oil - See Lubrication Diagram |
| Hydraulic cylinder bushings (8) | Apply light oil - See Lubrication Diagram |
| Inspect inboard roll stop for: |  |
| Wear or damage | Re-secure, replace or correct as needed. See Platform Angle instructions and Microswitch Adjustment Instructions. |
| Proper operation. Roll stop should just rest on top surface of the base plate. |  |
| Positive securement (both ends) |  |
| Inspect handrail components for wear or damage, and for proper operation | Replace damaged parts |
| Inspect microswitches for securement and proper adjustment | Re-secure, replace or adjust as needed. See Microswitch Adjustment Instructions |
| Make sure lift operates smoothly | Realign towers and vertical arms. Lubricate or correct as needed. |
| Inspect outboard roll stop clevis pin securement set screws | Re-secure or replace (apply Loctite 217). |
| Inspect external snap rings: |  |
| * Platform slide/rotate pivot pins (2 per pin) | Re-secure or replace as needed. |
| Platform fold axles (1 per axle) |  |
| Inboard roll stop lever bracket pins (1 per pin) |  |
| Lift-Tite™ latch gas (dampening)spring (2 per spring) |  |
| Outboard rolls top clevis pins(1 per pin) |  |
| Outboard roll stop foot pins (2) |  |
| Platform pivot pins (2) |  |
| Inspect platform fold axles and bearings for wear or damage and positive securement | Replace damaged parts and re-secure as needed. Apply Light Oil. |
| Inspect turnbuckle assemblies for wear or damage, proper operation and positive securement | Re-secure, replace or correct as needed. Apply light oil |
| Remove pump module cover and inspect: |  |
| Hydraulic hoses, fittings and connections for wear or leaks | Re-secure, replace or correct as needed. |
| Harness cables, wires, terminals and connections for securement or damage |  |
| Relays, fuses, circuit breakers and power switch for securement or damage |  |

Braun 4,500 Cycles

Perform all procedures listed in previous section

| Inspect cotter pins on platform pivot pins (2) | Re-secure, replace or correct as needed. |
| --- | --- |
| Hydraulic Fluid (Pump) - check level Note: Fluid should be changed if there is visible contamination. Inspect the hydraulic system (cylinder, hoses, fitting, seals, etc.) for leaks if fluid level is low. | Use Braun 32840-QT hydraulic fluid (Exxon® Univis HVI 26). Do not mix with Dextron III or other hydraulic fluids. Check fluid level with platform lowered fully. Fill to maximum fluid level indicated on reservoir (specified on decal). Do not overfill. If fluid level decal is not present - measure 35 mm from the fill port to locate fluid level. |
| Inspect cylinders, fitting and hydraulic connections for wear, damage or leaks | Tighten, repair or replace if needed. |
| Inspect parallel arm pivot pin mounting bolts (8) | Replace if needed. |
| Inspect platform pivot pin, bearings and vertical arms for wear, damage and positive securement | Replace damaged parts and Re-secure as needed. Apply Light Grease during reassembly procedures. |
| Inspect upper/lower fold arms, saddle, saddle support and associated pivot pins, bushings, and bearings for visible wear or damage. | Replace if needed. |
| Inspect gas springs (cylinders) for wear or damage, proper operation and positive securement | Tighten, replace or correct as needed |
| Inspect saddle bearings (buttons-4) | Apply Door-Ease or replace if needed. See Lubrication Diagram. |
| Inspect vertical arm plastic covers | Re-secure or replace as needed. |
| Inspect power cable | Re-secure, repair or replace as needed. |
| Mounting | Check to see that the lift is securely anchored to the vehicle and there are no loose bolts, broken welds or stress fractures |
| Decals and Anti-skid | Replace decals if worn, missing or illegible. Replace anti-skid if worn or missing. |

Lift Maintenance Schedule B - Ricon

Ricon 150 Cycles

| Over all condition | Listen for abnormal noises as lift operates: (i.e. grinding or binding noises) |
| --- | --- |
| Control Pendant | Verify that control pendant is undamaged and cable connector is tight. |
| Threshold warning system | Verify that system properly detects objects in threshold area and actuates the audible alarm. |
| Bridge plate load sensor | Verify that sensor inhibits downward movement of platform when a weight is present of lowered bridge plate. |
| Hydraulic fluids | Check for obvious hydraulic leaks |
| Electrical Wiring | Inspect electrical wiring for frayed wires, loose connectors, etc. |
| Vehicle interlock | Place vehicle in non-interlock mode and verify that lift does not operate. |
| Decals | Verify that lift decals are properly affixed, clearly visible and legible. Replace if necessary. |
| Armrests | Verify that armrest fasteners are properly tightened. |
| Lift mounting points | Verify that vehicle mounting and support points are undamaged |
|  | Verify that mounting bolts are sufficiently tight and free of corrosion |
| Main lifting pivots | Verify that link pins on arms are properly installed, free from damage, and locked in position |
| Platform pivot points | Verify that platform moves freely, without binding and does not wobble |
| Bridge plate | Verify that bridge plate operates without binding during lift functions |
|  | Verify that bridge plate deploys fully when platform stops at floor level |
|  | Verify that bridge plate rests flat against base plate. |
| Front roll stop | Verify that roll stop is opened completely when platform is at ground level |
|  | Verify that roll stop closes and locks when platform leaves ground |

Ricon 150 Cycles (Continued):

| Hydraulic Power unit | CAUTION |
| --- | --- |
|  | Check and add fluid when platform is at ground level. Fluid that is added when platform is raised will overflow when platform is lowered. |
|  | Verify that pump hydraulic fluid level is at FULL mark when platform is at ground level |
|  | Add Texaco 01554 Aircraft Hydraulic Oil or equivalent U.S. mil spec H5606G fluid |
|  | Verify there are no hydraulic fluid leaks |
|  | Verify that manual backup pump operates properly |

Ricon 1,800 Cycles

| Cleaning and lubrication |  |
| --- | --- |
|  | 1. Clean lift with mild soap and water and wipe dry. Prevent rust by coating all surfaces with light weight oil. Remove excess oil. |
|  | 2. Spray penetrating oil (Curtisol ® Red Grease 88167 or WD-40 ®) where specified following directions on container. Remove excess grease from surrounding areas. |

Ricon 3,600 Cycles

|  | CAUTION! |
| --- | --- |
|  | A Ricon authorized dealer must perform the following safety check. |
| Hydraulic cylinder hoses and fittings | Check hydraulic cylinder for evidence of leaks |
|  | Inspect hydraulic hoses for damage |
|  | Verify that all fittings are tight |
| Lift Maintenance Schedule C - Ramp |  |
| DAILY |  |
| Removable passenger seat | Be certain that removable passenger seat base is properly locked in position |
| Foldaway middle seat | Be certain that folding seat is locked firmly in position (either folded or unfolded) |
| MONTHLY |  |
| Sliding door | Wash lower door tracks and lightly lubricate contact surfaces |
| Folding ramp | Clean and light lubricate pivot points and hinges |
| SIX MONTH |  |
| Removable Passenger seat | Verify that locking mechanism on removable passenger seat operates properly by removing and reinstalling seat. |
| Electrical Connections (under hood) | Be certain that circuit breaker connections are free of corrosion; clean and apply protective coating as required. |
| ANNUAL |  |
| Under Carriage | Be certain undercoating is intact. Re-coat areas as required. Note: The rust inhibiting coating should provide years of trouble free service. However, severe condition (frequent use on unimproved or heavily salted road surfaces, etc.) may cause premature corrosion. Any area where undercoating is bulging due to flaky crust should be cleaned using a wire brush, treated with primer and re-coated. |
| Hoses and fittings | Be certain that all fuel lines are intact and not damaged. Be certain that fuel fill hoses are free of cracking and damage. |